Zone Voice Alarm Remote Microphone

EN-6500RM

Features:

- Digital voice evacuation system remote paging microphone.

- Each microphone of 12 zone capacity.

- System support 32 remote microphones of loop connection.

- System indicators of AC, DC, fault, Mic status and test.

- 12 zone selection buttons and all zone button.

- 12 zone selection buttons can be programmed.

- Two RJ45 ports for connection with EN-6500 and next remote microphone.

- One 3.5mm line input, with output volume control.

- Support PPT and normal paging two modes.

- CAT5 or CAT6 cable communication up to 600 meters.

- DC24V power supply from EVAC-500 controller.

- Support loop connection and paging microphone failure has been supervised.

Specifications:

| Model | EN-6500RM |
| --- | --- |
| Description | Voice Alarm Remote Microphone |
| Power Consumption | 3W |
| Zone Capacity | 12 Zones |
| System Capacity | 32 Remote microphones |
| Line Inputs | 350mV, 10kΩ |
| Mic Input | 5mV, 600Ω |
| Frequency Response | 80Hz~16kHz |
| THD | ＜1% at RMS,1KHz |
| S/N Ratio | >70dB |
| Operating Temp | -20℃ to +70℃ |
| Power Supply | DC24V, ±20% |
| Humidity | <95% no condensation |
| Dimension | 256(W)×149(H)×52(D) mm |
| Weight | 1.5kg |

Rear Panel: